# CPE403 – Advanced Embedded Systems

# Design Assignment 4

Name: Elmer Mejia

Email: mejiae4@unlv.nevada.edu

Github Repository link (root): assignments

Youtube Playlist link (root): Tiva_C

Follow the submission guideline to be awarded points for this Assignment.

- Block diagram and/or Schematics showing the components, pins used, and interface.

- Code for Tasks. for each task submit the modified or included code (from the base code) with highlights and justifications of the modifications. Also include the comments. If no base code is provided, submit the base code for the first task only. Use separate page for each task.

sunllghtService.c

/**********************************************************************************************

* Filename:       sunlightService.c

*

* Description:    This file contains the implementation of the service.

*

* Copyright (c) 2015-2019, Texas Instruments Incorporated

* All rights reserved.

*

* Redistribution and use in source and binary forms, with or without

* modification, are permitted provided that the following conditions

* are met:

*

* *  Redistributions of source code must retain the above copyright

*    notice, this list of conditions and the following disclaimer.

*

* *  Redistributions in binary form must reproduce the above copyright

*    notice, this list of conditions and the following disclaimer in the

*    documentation and/or other materials provided with the distribution.

*

* *  Neither the name of Texas Instruments Incorporated nor the names of

*    its contributors may be used to endorse or promote products derived

*    from this software without specific prior written permission.

*

* THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS "AS IS"

* AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO,

* THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

* PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT OWNER OR

* CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL,

* EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO,

* PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS;

* OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY,

* WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR

* OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE,

* EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

*

*************************************************************************************************/

/*********************************************************************

* INCLUDES

*/

#include <string.h>

#include <icall.h>

/* This Header file contains all BLE API and icall structure definition */

#include "icall_ble_api.h"

#include "sunlightService.h"

/*********************************************************************

* MACROS

*/

/*********************************************************************

* CONSTANTS

*/

/*********************************************************************

* TYPEDEFS

*/

/*********************************************************************

* GLOBAL VARIABLES

*/

// sunlightService Service UUID

CONST uint8_t sunlightServiceUUID[ATT_BT_UUID_SIZE] =

{

LO_UINT16(SUNLIGHTSERVICE_SERV_UUID), HI_UINT16(SUNLIGHTSERVICE_SERV_UUID)

};

// sunlightValue UUID

CONST uint8_t sunlightService_SunlightValueUUID[ATT_UUID_SIZE] =

{

TI_BASE_UUID_128(SUNLIGHTSERVICE_SUNLIGHTVALUE_UUID)

};

// updatePeriod UUID

CONST uint8_t sunlightService_UpdatePeriodUUID[ATT_UUID_SIZE] =

{

TI_BASE_UUID_128(SUNLIGHTSERVICE_UPDATEPERIOD_UUID)

};

/*********************************************************************

* LOCAL VARIABLES

*/

static sunlightServiceCBs_t *pAppCBs = NULL;

/*********************************************************************

* Profile Attributes - variables

*/

// Service declaration

static CONST gattAttrType_t sunlightServiceDecl = { ATT_BT_UUID_SIZE, sunlightServiceUUID };

// Characteristic "SunlightValue" Properties (for declaration)

static uint8_t sunlightService_SunlightValueProps = GATT_PROP_NOTIFY;

// Characteristic "SunlightValue" Value variable

static uint8_t sunlightService_SunlightValueVal[SUNLIGHTSERVICE_SUNLIGHTVALUE_LEN] = {0};

// Characteristic "SunlightValue" CCCD

static gattCharCfg_t *sunlightService_SunlightValueConfig;

// Characteristic "UpdatePeriod" Properties (for declaration)

static uint8_t sunlightService_UpdatePeriodProps = GATT_PROP_READ | GATT_PROP_WRITE;

// Characteristic "UpdatePeriod" Value variable

static uint8_t sunlightService_UpdatePeriodVal[SUNLIGHTSERVICE_UPDATEPERIOD_LEN] = {0};

/*********************************************************************

* Profile Attributes - Table

*/

static gattAttribute_t sunlightServiceAttrTbl[] =

{

// sunlightService Service Declaration

{

{ ATT_BT_UUID_SIZE, primaryServiceUUID },

GATT_PERMIT_READ,

0,

(uint8_t *)&sunlightServiceDecl

},

// SunlightValue Characteristic Declaration

{

{ ATT_BT_UUID_SIZE, characterUUID },

GATT_PERMIT_READ,

0,

&sunlightService_SunlightValueProps

},

// SunlightValue Characteristic Value

{

{ ATT_UUID_SIZE, sunlightService_SunlightValueUUID },

GATT_PERMIT_READ,

0,

sunlightService_SunlightValueVal

},

// SunlightValue CCCD

{

{ ATT_BT_UUID_SIZE, clientCharCfgUUID },

GATT_PERMIT_READ | GATT_PERMIT_WRITE,

0,

(uint8 *)&sunlightService_SunlightValueConfig

},

// UpdatePeriod Characteristic Declaration

{

{ ATT_BT_UUID_SIZE, characterUUID },

GATT_PERMIT_READ,

0,

&sunlightService_UpdatePeriodProps

},

// UpdatePeriod Characteristic Value

{

{ ATT_UUID_SIZE, sunlightService_UpdatePeriodUUID },

GATT_PERMIT_READ | GATT_PERMIT_WRITE,

0,

sunlightService_UpdatePeriodVal

},

};

/*********************************************************************

* LOCAL FUNCTIONS

*/

static bStatus_t sunlightService_ReadAttrCB( uint16_t connHandle, gattAttribute_t *pAttr,

uint8_t *pValue, uint16_t *pLen, uint16_t offset,

uint16_t maxLen, uint8_t method );

static bStatus_t sunlightService_WriteAttrCB( uint16_t connHandle, gattAttribute_t *pAttr,

uint8_t *pValue, uint16_t len, uint16_t offset,

uint8_t method );

/*********************************************************************

* PROFILE CALLBACKS

*/

// Simple Profile Service Callbacks

CONST gattServiceCBs_t sunlightServiceCBs =

{

sunlightService_ReadAttrCB,  // Read callback function pointer

sunlightService_WriteAttrCB, // Write callback function pointer

NULL                       // Authorization callback function pointer

};

/*********************************************************************

* PUBLIC FUNCTIONS

*/

/*

* SunlightService_AddService- Initializes the SunlightService service by registering

*          GATT attributes with the GATT server.

*

*/

extern bStatus_t SunlightService_AddService( uint8_t rspTaskId )

{

uint8_t status;

// Allocate Client Characteristic Configuration table

sunlightService_SunlightValueConfig = (gattCharCfg_t *)ICall_malloc( sizeof(gattCharCfg_t) * linkDBNumConns );

if ( sunlightService_SunlightValueConfig == NULL )

{

return ( bleMemAllocError );

}

// Initialize Client Characteristic Configuration attributes

GATTServApp_InitCharCfg( LINKDB_CONNHANDLE_INVALID, sunlightService_SunlightValueConfig );

// Register GATT attribute list and CBs with GATT Server App

status = GATTServApp_RegisterService( sunlightServiceAttrTbl,

GATT_NUM_ATTRS( sunlightServiceAttrTbl ),

GATT_MAX_ENCRYPT_KEY_SIZE,

&sunlightServiceCBs );

return ( status );

}

/*

* SunlightService_RegisterAppCBs - Registers the application callback function.

*                    Only call this function once.

*

*    appCallbacks - pointer to application callbacks.

*/

bStatus_t SunlightService_RegisterAppCBs( sunlightServiceCBs_t *appCallbacks )

{

if ( appCallbacks )

{

pAppCBs = appCallbacks;

return ( SUCCESS );

}

else

{

return ( bleAlreadyInRequestedMode );

}

}

/*

* SunlightService_SetParameter - Set a SunlightService parameter.

*

*    param - Profile parameter ID

*    len - length of data to right

*    value - pointer to data to write.  This is dependent on

*          the parameter ID and WILL be cast to the appropriate

*          data type (example: data type of uint16 will be cast to

*          uint16 pointer).

*/

bStatus_t SunlightService_SetParameter( uint8_t param, uint16_t len, void *value )

{

bStatus_t ret = SUCCESS;

switch ( param )

{

case SUNLIGHTSERVICE_SUNLIGHTVALUE_ID:

if ( len == SUNLIGHTSERVICE_SUNLIGHTVALUE_LEN )

{

memcpy(sunlightService_SunlightValueVal, value, len);

// Try to send notification.

GATTServApp_ProcessCharCfg( sunlightService_SunlightValueConfig, (uint8_t *)&sunlightService_SunlightValueVal, FALSE,

sunlightServiceAttrTbl, GATT_NUM_ATTRS( sunlightServiceAttrTbl ),

INVALID_TASK_ID,  sunlightService_ReadAttrCB);

}

else

{

ret = bleInvalidRange;

}

break;

case SUNLIGHTSERVICE_UPDATEPERIOD_ID:

if ( len == SUNLIGHTSERVICE_UPDATEPERIOD_LEN )

{

memcpy(sunlightService_UpdatePeriodVal, value, len);

}

else

{

ret = bleInvalidRange;

}

break;

default:

ret = INVALIDPARAMETER;

break;

}

return ret;

}

/*

* SunlightService_GetParameter - Get a SunlightService parameter.

*

*    param - Profile parameter ID

*    value - pointer to data to write.  This is dependent on

*          the parameter ID and WILL be cast to the appropriate

*          data type (example: data type of uint16 will be cast to

*          uint16 pointer).

*/

bStatus_t SunlightService_GetParameter( uint8_t param, uint16_t *len, void *value )

{

bStatus_t ret = SUCCESS;

switch ( param )

{

case SUNLIGHTSERVICE_UPDATEPERIOD_ID:

memcpy(value, sunlightService_UpdatePeriodVal, SUNLIGHTSERVICE_UPDATEPERIOD_LEN);

break;

default:

ret = INVALIDPARAMETER;

break;

}

return ret;

}

/*********************************************************************

* @fn          sunlightService_ReadAttrCB

*

* @brief       Read an attribute.

*

* @param       connHandle - connection message was received on

* @param       pAttr - pointer to attribute

* @param       pValue - pointer to data to be read

* @param       pLen - length of data to be read

* @param       offset - offset of the first octet to be read

* @param       maxLen - maximum length of data to be read

* @param       method - type of read message

*

* @return      SUCCESS, blePending or Failure

*/

static bStatus_t sunlightService_ReadAttrCB( uint16_t connHandle, gattAttribute_t *pAttr,

uint8_t *pValue, uint16_t *pLen, uint16_t offset,

uint16_t maxLen, uint8_t method )

{

bStatus_t status = SUCCESS;

// See if request is regarding the SunlightValue Characteristic Value

if ( ! memcmp(pAttr->type.uuid, sunlightService_SunlightValueUUID, pAttr->type.len) )

{

if ( offset > SUNLIGHTSERVICE_SUNLIGHTVALUE_LEN )  // Prevent malicious ATT ReadBlob offsets.

{

status = ATT_ERR_INVALID_OFFSET;

}

else

{

*pLen = MIN(maxLen, SUNLIGHTSERVICE_SUNLIGHTVALUE_LEN - offset);  // Transmit as much as possible

memcpy(pValue, pAttr->pValue + offset, *pLen);

}

}

// See if request is regarding the UpdatePeriod Characteristic Value

else if ( ! memcmp(pAttr->type.uuid, sunlightService_UpdatePeriodUUID, pAttr->type.len) )

{

if ( offset > SUNLIGHTSERVICE_UPDATEPERIOD_LEN )  // Prevent malicious ATT ReadBlob offsets.

{

status = ATT_ERR_INVALID_OFFSET;

}

else

{

*pLen = MIN(maxLen, SUNLIGHTSERVICE_UPDATEPERIOD_LEN - offset);  // Transmit as much as possible

memcpy(pValue, pAttr->pValue + offset, *pLen);

}

}

else

{

// If we get here, that means you've forgotten to add an if clause for a

// characteristic value attribute in the attribute table that has READ permissions.

*pLen = 0;

status = ATT_ERR_ATTR_NOT_FOUND;

}

return status;

}

/*********************************************************************

* @fn      sunlightService_WriteAttrCB

*

* @brief   Validate attribute data prior to a write operation

*

* @param   connHandle - connection message was received on

* @param   pAttr - pointer to attribute

* @param   pValue - pointer to data to be written

* @param   len - length of data

* @param   offset - offset of the first octet to be written

* @param   method - type of write message

*

* @return  SUCCESS, blePending or Failure

*/

static bStatus_t sunlightService_WriteAttrCB( uint16_t connHandle, gattAttribute_t *pAttr,

uint8_t *pValue, uint16_t len, uint16_t offset,

uint8_t method )

{

bStatus_t status  = SUCCESS;

uint8_t   paramID = 0xFF;

// See if request is regarding a Client Characteristic Configuration

if ( ! memcmp(pAttr->type.uuid, clientCharCfgUUID, pAttr->type.len) )

{

// Allow only notifications.

status = GATTServApp_ProcessCCCWriteReq( connHandle, pAttr, pValue, len,

offset, GATT_CLIENT_CFG_NOTIFY);

}

// See if request is regarding the UpdatePeriod Characteristic Value

else if ( ! memcmp(pAttr->type.uuid, sunlightService_UpdatePeriodUUID, pAttr->type.len) )

{

if ( offset + len > SUNLIGHTSERVICE_UPDATEPERIOD_LEN )

{

status = ATT_ERR_INVALID_OFFSET;

}

else

{

// Copy pValue into the variable we point to from the attribute table.

memcpy(pAttr->pValue + offset, pValue, len);

// Only notify application if entire expected value is written

if ( offset + len == SUNLIGHTSERVICE_UPDATEPERIOD_LEN)

paramID = SUNLIGHTSERVICE_UPDATEPERIOD_ID;

}

}

else

{

// If we get here, that means you've forgotten to add an if clause for a

// characteristic value attribute in the attribute table that has WRITE permissions.

status = ATT_ERR_ATTR_NOT_FOUND;

}

// Let the application know something changed (if it did) by using the

// callback it registered earlier (if it did).

if (paramID != 0xFF)

if ( pAppCBs && pAppCBs->pfnChangeCb )

pAppCBs->pfnChangeCb(connHandle, paramID, len, pValue); // Call app function from stack task context.

return status;

}

Sunlight service.h

/**********************************************************************************************

* Filename:       sunlightService.h

*

* Description:    This file contains the sunlightService service definitions and

*                 prototypes.

*

* Copyright (c) 2015-2019, Texas Instruments Incorporated

* All rights reserved.

*

* Redistribution and use in source and binary forms, with or without

* modification, are permitted provided that the following conditions

* are met:

*

* *  Redistributions of source code must retain the above copyright

*    notice, this list of conditions and the following disclaimer.

*

* *  Redistributions in binary form must reproduce the above copyright

*    notice, this list of conditions and the following disclaimer in the

*    documentation and/or other materials provided with the distribution.

*

* *  Neither the name of Texas Instruments Incorporated nor the names of

*    its contributors may be used to endorse or promote products derived

*    from this software without specific prior written permission.

*

* THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS "AS IS"

* AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO,

* THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

* PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT OWNER OR

* CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL,

* EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO,

* PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS;

* OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY,

* WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR

* OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE,

* EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

*

*************************************************************************************************/

#ifndef _SUNLIGHTSERVICE_H_

#define _SUNLIGHTSERVICE_H_

#ifdef __cplusplus

extern "C"

{

#endif

/*********************************************************************

* INCLUDES

*/

/*********************************************************************

* CONSTANTS

*/

#ifdef DeviceFamily_CC26X0R2

#define LINKDB_CONNHANDLE_INVALID CONNHANDLE_INVALID

#endif //DeviceFamily_CC26X0R2

// Service UUID

#define SUNLIGHTSERVICE_SERV_UUID 0xBA55

//  Characteristic defines

#define SUNLIGHTSERVICE_SUNLIGHTVALUE_ID   0

#define SUNLIGHTSERVICE_SUNLIGHTVALUE_UUID 0x2BAD

#define SUNLIGHTSERVICE_SUNLIGHTVALUE_LEN  4

//  Characteristic defines

#define SUNLIGHTSERVICE_UPDATEPERIOD_ID   1

#define SUNLIGHTSERVICE_UPDATEPERIOD_UUID 0x2BAE

#define SUNLIGHTSERVICE_UPDATEPERIOD_LEN  2

/*********************************************************************

* TYPEDEFS

*/

/*********************************************************************

* MACROS

*/

/*********************************************************************

* Profile Callbacks

*/

// Callback when a characteristic value has changed

typedef void (*sunlightServiceChange_t)(uint16_t connHandle, uint8_t paramID, uint16_t len, uint8_t *pValue);

typedef struct

{

sunlightServiceChange_t        pfnChangeCb;  // Called when characteristic value changes

sunlightServiceChange_t        pfnCfgChangeCb;

} sunlightServiceCBs_t;

/*********************************************************************

* API FUNCTIONS

*/

/*

* SunlightService_AddService- Initializes the SunlightService service by registering

*          GATT attributes with the GATT server.

*

*/

extern bStatus_t SunlightService_AddService( uint8_t rspTaskId);

/*

* SunlightService_RegisterAppCBs - Registers the application callback function.

*                    Only call this function once.

*

*    appCallbacks - pointer to application callbacks.

*/

extern bStatus_t SunlightService_RegisterAppCBs( sunlightServiceCBs_t *appCallbacks );

/*

* SunlightService_SetParameter - Set a SunlightService parameter.

*

*    param - Profile parameter ID

*    len - length of data to right

*    value - pointer to data to write.  This is dependent on

*          the parameter ID and WILL be cast to the appropriate

*          data type (example: data type of uint16 will be cast to

*          uint16 pointer).

*/

extern bStatus_t SunlightService_SetParameter(uint8_t param, uint16_t len, void *value);

/*

* SunlightService_GetParameter - Get a SunlightService parameter.

*

*    param - Profile parameter ID

*    value - pointer to data to write.  This is dependent on

*          the parameter ID and WILL be cast to the appropriate

*          data type (example: data type of uint16 will be cast to

*          uint16 pointer).

*/

extern bStatus_t SunlightService_GetParameter(uint8_t param, uint16_t *len, void *value);

/*********************************************************************

*********************************************************************/

#ifdef __cplusplus

}

#endif

#endif /* _SUNLIGHTSERVICE_H_ */

Screenshots of the IDE, physical setup, debugging process - Provide screenshot of successful compilation, screenshots of registers, variables, graphs, etc.

- Declaration

I understand the Student Academic Misconduct Policy - http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

-Elmer Mejia